X6 Access Control

System User Manual

1.Equipment Installation

Wall mount installation

(1)Remove the  screw on the    (2)Take away the back cover.

bottom of device.

(3)Fix the back cover  on the

wall

according

to   the

mounting paper.

(4)Fix the device to the back     (5)Fix the screw.

cover.

2.Structure and Function

Access Control System Function:

(1)If  a  registered  user  verified,  the  device  will

export the signal to unlock the door.

(2)Door sensor  will detect the  on-off state.  If the

door is  unexpected opened  or improperly  closed,

the alarm signal (digital value) will be triggered.

(3)If the device being illegally removed, the device

will export alarm signal.

(4)External exit button is supported; it is convenient

to open the door inside.

(5)External bell is supported.

1

3.Lock  Connection

Warning: No operation when power on!

(1) The system supports NO LOCK and NC LOCK.  For example the NO LOCK

(normally open at power on) is connected with’NO’ terminal, and the NC LOCK is

connected with ‘NC’ terminal.

(2) When the Electrical Lock is connected to the Access Control System, you need

to

parallel  one   FR107   diode  (equipped   in  the   package)  to   prevent   the

self-inductance EMF affect the system, do not reverse the polarities.

(I)Share power with the lock:

Deviceshare power with the

lock:

ULOCK=12V,

I-ILOCK>1A……

1

Andthe lockis near to the

device.

(II)Does not share power with the lock:

Devicedoes notsharepower

with thelock:

A.ULOCK =12V

≤1A;

I-ILOCK

B.ULOCK ≠12V;

C.The lock is far apart fromthe

device.

I①:   ‘I’: device output current, ‘ULOCK’: lock voltage,’ ILOCK’: lock current.

2

3

4. Conect with alarm

Notice: Voltage output ≤ DC 12V for Alarm

5.Connect with Power

Input DC 12V, 500mA (50mA standby) Positive is  connected with‘+12V, negative is connected with’GND’(do   not  reverse the polarities

4

Instructions

Buttons represent: [#] means yes, [* ]means Escape

Lights represent: Green means sucess, Red means Failure

Number: Password=4 digits, work code<=5 digits

Tones: Long buzz means success, short buzz twice means failure, short buzz 4 times means wrong action

Notice: make sure you pressed [#] to confirm, [*] exit to initial condition

1. User management

  - Administraction Operations

For the security the data, admin password is needed to operate the machine.

- Register the admin's fingerprint

Press [*]+[#]→Input [1234#]→register fingerprint  3 times (green light on and a long buzz) →press [*] to exit (green light on and a long buzz)

- Modify the admin's fingerprint

Press [*]+[#]→Press admin fingerprint→press [8] (green light on and a long buzz) →register fingerprint 3 times → press [*] to exit (green light on and a long buzz)

- Lost the admin fingerprint

Please contact customer service at: 4006-900-999

  - Add/Edit a User

Each user can regist 10 fingeprint and 1 card at most

Press [*]+[#]→ress admin fingerprint→press [#]→ Input a new/ exist work code [#]→ register fingerprint  3 times (green light on and a long buzz) →press [#] contiue to add new users  OR press [*] to escape(green light on and a long buzz)

Notice:

- New user has to put work code,the machine will defultly edit the present  user if the number existed.

- After the first fingeprint registered, same user's other fingerprints can be added, 10 pcs at most

- If there is wrong action or repeat occured suring register fingeprint/ cards, machine's red light will be on along with short buzz 4 times, then green light will be on. Continue to register without exit, no operation limits.

  - Register Eight Passwords for Opening the Door

The machine support at most 8groups of door password, each password respond to a group (range: 1~8):

Press [*]+[#]→Press admin fingerprint→press [3] →Input modified group 　number (1~8) → Put 4 digits password [#] (repeat twice) →press [*] to exit

Notice:

- When a group of password was changed into 00000, then this group of password was shut down

- When a group of password was changed successfully, you can input next group of number to modify directly.

  - User Authentication

Fingerprint / Card / Password authentication

Connect the power, the machine is on the mode of authentication.

Press password [#] / Press fingerprint / Swipe card → Authenticat successfully

Notice:

When use a password , the door can be opened as long as the password is same as one of the 8 groups of passwords. Original 8 groups of password are empty.

  - Delete Users

- Delete a single user

Press [*]+[#]→Press admin fingerprint→press [3] →Press [2] → Put work number [#] / fingerprint / card →press [*] to exit

- Delete all of the users

Press [*]+[#]→Press admin fingerprint→press [9]+[#]→press [*] to exit

2. Acess control management (Default setting is recommened)

  - Configure Unlocking Duration (Default 6 seconds)

Press [*]+[#]→Press admin fingerprint→press [4] →Input unlock duration        (1s~10s) →press [*] to exit

When the unlock duration is bigger than 10, red light will be on.

  - Configure Authentication Mode (FP/PW/RF)

Press [*]+[#]→Press admin fingerprint→press [5] →Input authentication mode value [#]→press [*] to exit

Authentication Mode Value Description:

| Mode | Value | Description |
| --- | --- | --- |
| PW | 1 | Only Password |
| RF | 2 | Only Card |
| FP | 3 | Only fingerprint |
| FP/PW/RF | 4 | Fingerprint or password or card |
| RF& PW | 5 | Card then password |
| FP & PW | 6 | Fingeprint them password |

  - Configure Alarm

- Alarm Switch (Default ON)

The switch is the master switch of the machine, only when it's on, can the alarm incorrectly, tamper alarm and magnet alarm be working properly. The alarm will be shut dowm when it's off.

Press [*]+[#]→Press admin fingerprint→press [0]+ [1]→Configure main switch [#] (0: ON, 1:OFF)→press [*] to exit

- Tamper Alarm Configure (Default ON)

Only when this function is on, the machine can be alarmed during tampering.

Press [*]+[#]→Press admin fingerprint→press [7]→Configure tamper alarm switch [#] (0: ON, 1:OFF)→press [*] to exit